MOBILE ASPECTS

Cabinet Status Management System

Technical Reference Documentation

| Document Version | Version 1.0.0(1) |
| --- | --- |
| Date | May 2026 |
| Initiated By | M. Rahim |
| Organization | Mobile Aspects |
| Classification | Confidential – Internal Use Only |
| Status | Draft |

# 1. Objective

This document outlines the procedure for managing cabinet status using the React application and RabbitMQ messaging protocol via the WebSocket broker. It is intended to serve as the definitive technical reference for developers, administrators, and operators within Mobile Aspects who are responsible for the deployment, integration, and ongoing monitoring of the cabinet management system.

# 2. Scope

This document applies to all Mobile Aspects personnel and developers responsible for the deployment, maintenance, and monitoring of the cabinet status system through the React application. The WebSocket broker is hosted at:

ws://brokeraddress:15675/ws

# 3. Pre-requisites

Before proceeding with configuration and deployment, ensure the following prerequisites are satisfied:

- The WebSocket MQTT broker (ws://brokeraddress:15675/ws) is fully operational.

- React environment variables are correctly configured in the .env file.

- Access to the source code repository has been granted.

- Node.js and npm/yarn are installed on the development machine.

3.1  Environment Configuration

Add the following entries to the project .env file. Restart the React application after any updates to this file.

# ===================================== # RabbitMQ WebSocket & Credentials # ===================================== REACT_APP_RABBIT_WS=ws://192.168.3.89:15675/ws REACT_APP_RABBIT_USER=guest REACT_APP_RABBIT_PASS=guest

3.2  Cabinet MAC Address Configuration

For each cabinet, enter the MAC address value in the REACT_APP_MAC_ADDRESS variable. Multiple addresses are comma-separated:

REACT_APP_MAC_ADDRESS=MASCAB01A,MASCAB01B

3.3  Backend Mapping Configuration

Configure the backend mapping for each cabinet, specifying whether it communicates via RabbitMQ or Azure:

REACT_APP_BACKEND_MAP=MASCAB01A=rabbit,MASCAB01B=azure

# 4. Responsibilities

| Role | Responsibility |
| --- | --- |
| Developers | Implement, test, and debug the React application and MQTT integration. |
| Administrators | Monitor the RabbitMQ broker, manage credentials, and ensure data integrity. |
| Operators | Execute commands via the application and monitor cabinet statuses in real time. |

# 5. Configuration & Deployment Procedure

Step 1 — Configure MQTT Connection

Open the CabinetStatusComponent React component file and ensure the RABBIT_WS constant references the correct broker URL:

const RABBIT_WS = process.env.REACT_APP_RABBIT_WS ¦¦ "ws://192.168.3.89:15675/ws";

Step 2 — Subscribe to Topics

Verify that the application subscribes to the following MQTT topics:

| Topic | Description |
| --- | --- |
| in | Inbound command channel — commands sent to cabinet devices. |
| out | Outbound data channel — status and inventory data received from cabinets. |
| ${mac}/reboot | Per-device reboot notification channel (replace ${mac} with device MAC address). |

Step 3 — Handle Incoming Messages

Confirm the client.on('message') function correctly processes data for all subscribed topics. The handler must:

- Update cabinet status from the out topic.

- Process inventory data received on the out topic.

- Read and update lock statuses from the in topic.

- Read and update light statuses from the in topic.

- Update application state using setCabinets and setDoorStatuses accordingly.

Step 4 — Send Commands

Use the sendCommand function to issue the following commands to cabinet devices:

| Command | Payload (published to topic in) |
| --- | --- |
| Start Scan (single) | {"rfid":"Startscan","sessionId":"","mac":"MASCAB01A"} |
| Start Scan (all) | {"rfid":"Startscan","sessionId":"","mac":"all"} |
| Stop Scan (single) | {"rfid":"Stopscan","sessionId":"testlagi","mac":"MASCAB01A"} |
| Stop Scan (all) | {"rfid":"Stopscan","sessionId":"testlagi","mac":"all"} |

Step 5 — Deploy and Monitor

Run the React development server with the following command:

npm run dev

Monitor the browser console output to verify MQTT messages are being received and that topics are being processed correctly.

# 6. Command Flow Reference

This section documents all supported RabbitMQ commands and their corresponding payloads. Commands are published via the RabbitMQ Management UI (Exchanges tab > Publish Message) or programmatically via the application.

6.1  Door Status

Door status messages are received on the out topic from the device.

| Event | Topic | Payload |
| --- | --- | --- |
| Door Open | out | {"cmd":"Open","sessionId":"testlagi","mac":"MASCAB01A”} |
| Door Close | out | {"cmd":"Close","sessionId":"testlagi","mac":"MASCAB01A"} |

6.2  Cabinet Status

A full cabinet status message is received on the out topic, containing door, light, lock, IP, and timing information:

{ "mac": "MASCAB01A", "hostname": "SUPPLY7", "sessionId": "", "time": "2026-02-10 08:55:27", “Door1”: "Close", "DOOR2": "Open", “Light”: "On", “Lock1”: "Lock", "IP_ETH0": "192.168.137.239", "IP_WLAN0": "0.0.0.0" }

6.3  Session Management

| Command | Topic | Payload |
| --- | --- | --- |
| Start Session | in | {"cmd":"Start","sessionId":"testlagi","ip":"192.168.137.101"} |
| Stop Session | in | {"cmd":"Stopsession","sessionId":"testlagi","ip":"192.168.137.101"} |

6.4  RFID Scan Control

| Command | Topic | Payload |
| --- | --- | --- |
| Start Scan (single) | in | {"rfid":"Startscan","sessionId":"","mac":"MASCAB01A"} |
| Start Scan (all) | in | {"rfid":"Startscan","sessionId":"","mac":"all"} |
| Stop Scan (single) | in | {"rfid":"Stopscan","sessionId":"testlagi","mac":"MASCAB01A"} |
| Stop Scan (all) | in | {"rfid":"Stopscan","sessionId":"testlagi","mac":"all"} |

6.5  Lock / Unlock

| Command | Topic | Payload |
| --- | --- | --- |
| Lock All Doors | in | {"lock":"Lock","sessionId":"testlagi","mac":"all"} |
| Unlock All Doors | in | {"lock":"Unlock","sessionId":"testlagi","mac":"all"} |

6.6  Lighting Control

| Command | Topic | Payload |
| --- | --- | --- |
| Light ON (all) | in | {“Light”:"On","sessionId":"","mac":"all"} |
| Light OFF (all) | in | {“Light”:"Off","sessionId":"","mac":"all"} |

6.7  Reboot

| Command | Topic | Payload |
| --- | --- | --- |
| Reboot Cabinet | in | {"cmd":"Reboot","sessionId":"","mac":"MASCAB01A"} |

6.8  Session ID

The following commands update a device's config.json file. Published to the in topic.

| Command | Topic | Payload |
| --- | --- | --- |
| Session Id All | in | {"cmd":"Start","sessionId":"test","ip":"all"} |
| Session Id Per cabinet | in | {"cmd":"Start","sessionId":"test","ip":"192.168.137.101"} |

6.9  Status Now (Beacon)

The following commands update a device's config.json file. Published to the in topic.

| Command | Topic | Payload |
| --- | --- | --- |
| Nowstatus | in | {"cmd":"NowStatus","sessionId":"test","mac":"all"} |

6.10  Inventory Output (OUTJSON)

After a scan session, inventory data is published by the device to the out topic. The payload includes cabinet metadata, RFID tag inventory, and change sets:

{ "macaddress": "MASCAB01A", "hostname": "RAP1", "sessionId": "testlagi", "time": "2026-02-10 09:04:37", "duration": 15, "count": 118, “Light”: "On", “Lock1”: "Lock", "inventory": ["645349000000002B27BEF141", "..."], "add": [], "remove": [], "mac": "MASCAB01A" }

# 7. Command Quick Reference Summary

| Command | Topic | Payload |
| --- | --- | --- |
| Door Open (device) | out | {"cmd":"Open","sessionId":"test","mac":"MASCAB01A"} |
| Door Close (device) | out | {"cmd":"Close","sessionId":"test,"mac":"MASCAB01A"} |
| Start Session | in | {"cmd":"Start","sessionId":"test","ip":"192.168.137.101"} |
| - Response | out | {"mac": "MASCAB01A", "hostname": "MASCAB01A", "info": "Session test started", "sessionId": "test", "count": 73, "duration": 11, "status": "ok"} |
|  | out | {"mac":"MASCAB01A","macAddress":"b8:27:eb:92:cd:87", "hostname":"MASCAB01A","sessionId":"test","time":"2026-05-05 15:09:08",”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", "IP_ETH0":"192.168.137.101"} |
| Start Session(all) | in | {"cmd":"Start","sessionId":"test","ip":"all"} |
|  | out | {"mac": "MASCAB01A", "hostname": "MASCAB01A", "info": "Session test started", "sessionId": "test", "count": 73, "duration": 11, "status": "ok"} |
|  | out | {"mac":"MASCAB01A","macAddress":" b8:27:eb:92:cd:87","hostname":"MASCAB01A", "sessionId":"test","time":"2026-05-05 18:40:50",”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", "IP_ETH0":"192.168.137.101"} |
|  | out | {"mac": "MASCAB01B", "hostname": "MASCAB01B", "info": "Session test started", "sessionId": "test", "count": 73, "duration": 11, "status": "ok"} |
|  | out | {"mac":"MASCAB01B","macAddress":" b8:27:eb:6b:db:1e","hostname":"MASCAB01B", "sessionId":"test","time":"2026-05-05 18:40:50",”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", "IP_ETH0":"192.168.137.102"} |
| Stop Session | in | {"cmd":"Stopsession","sessionId":"test","ip":"192.168.137.101"} |
| - Response | out | {"mac": "MASCAB01A", "hostname": "MASCAB01A", "info": "Session stopped", "stoppedSession": "", "sessionId": "", "status": "ok"} |
|  | out | {"mac":"MASCAB01A","macAddress":"b8:27:eb:92:cd:87", "hostname":"MASCAB01A","sessionId":"","time":"2026-05-05 15:09:51",”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", "IP_ETH0":"192.168.137.101"} |
| Stop Session (all) | in | {"cmd":"Stopsession","sessionId":"test","ip":"all"} |
|  | out | {"mac": "MASCAB01A", "hostname": "MASCAB01A", "info": "Session stopped", "stoppedSession": "test", "sessionId": "", "status": "ok"} |
|  | out | {"mac":"MASCAB01A","macAddress":"b8:27:eb:92:cd:87", "hostname":"MASCAB01A","sessionId":"","time":"2026-05-05 15:09:51",”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", "IP_ETH0":"192.168.137.101"} |
|  | out | {"mac": "MASCAB01B", "hostname": "MASCAB01B", "info": "Session stopped", "stoppedSession": "test", "sessionId": "", "status": "ok"} |
|  | out | {"mac":"MASCAB01B","macAddress":"b8:27:eb:6b:db:1e", "hostname":"MASCAB01B","sessionId":"","time":"2026-05-05 15:09:51",”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", "IP_ETH0":"192.168.137.102"} |
| Start Scan (single) | in | {"rfid":"Startscan","sessionId":"test","mac":"MASCAB01A"} |
| - Response | out | {"cmd":"Scanning","hostname":"MASCAB01A","mac":"MASCAB01A","sessionId":"test"} |
|  | out | {"macaddress":"b8:27:eb:92:cd:87","hostname":"MASCAB01A","IP_ETH0":"192.168.137.101", "sessionId":"test","time":"2026-05-05 15:13:00", "duration":12,"count":73,”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", "inventory":["8600153329000000000007A0",”xxxxx”],"add":[], "remove":[],"mac":"MASCAB01A"} |
| Start Scan (all) | in | {"rfid":"Startscan","sessionId":"test","mac":"all"} |
| - Response | out | {"cmd":"Scanning","hostname":"MASCAB01A","mac":"MASCAB01A","sessionId":"test"} |
|  |  | {"macaddress":"b8:27:eb:92:cd:87","hostname":"MASCAB01A","IP_ETH0":"192.168.137.101", "sessionId":"test","time":"2026-05-05 15:13:00", "duration":12,"count":73,”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", "inventory":["8600153329000000000007A0",”xxxxx”],"add":[], "remove":[],"mac":"MASCAB01A"} |
| Stop Scan (single) | in | {"rfid":"Stopscan","sessionId":"test","mac":"MASCAB01A"} |
| - Response | out | {"macaddress":"b8:27:eb:92:cd:87","hostname":"MASCAB01A","IP_ETH0":"192.168.137.101", "sessionId":"test","time":"2026-05-05 15:13:00", "duration":12,"count":73,”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", “mac":"MASCAB01A"} |
| Stop Scan (all) | in | {"rfid":"Stopscan","sessionId":"test","mac":"all"} |
| - Response | out | {"macaddress":"b8:27:eb:92:cd:87","hostname":"MASCAB01A","IP_ETH0":"192.168.137.101", "sessionId":"test","time":"2026-05-05 15:13:00", "duration":12,"count":73,”Door1”:"Close",”Light”:"On",”Lock1”:"Lock", "mac":"MASCAB01A"} |
| Lock All Doors | in | {"lock":"Lock","sessionId":"test","mac":"all"} |
| - Response | out | {"mac":"MASCAB01A","macAddress":"b8:27:eb:92:cd:87","hostname":"MASCAB01A", "sessionId":"test","time":"2026-05-05 15:21:50", “Door1”:"Close",”Light”:"On",”Lock1”:"Lock","IP_ETH0":"192.168.137.101"} |
| Unlock All Doors | in | {"lock":"Unlock","sessionId":"test","mac":"all"} |
| - Response | out | {"mac":"MASCAB01A","macAddress":"b8:27:eb:92:cd:87","hostname":"MASCAB01A", "sessionId":"test","time":"2026-05-05 15:22:36", “Door1”:"Close",”Light”:"On",”Lock1”:"Unlock","IP_ETH0":"192.168.137.101"} |
| Light ON (all) | in | {“Light”:"On","sessionId":"test","mac":"all"} |
| - Response | out | {"mac":"MASCAB01A","macAddress":"b8:27:eb:92:cd:87","hostname":"MASCAB01A", "sessionId":"test","time":"2026-05-05 15:23:46", “Door1”:"Close",”Light”:"On",”Lock1”:"Unlock","IP_ETH0":"192.168.137.101"} |
| Light OFF (all) | in | {“Light”:"Off","sessionId":"test","mac":"all"} |
| - Response | out | {"mac":"MASCAB01A","macAddress":"b8:27:eb:92:cd:87","hostname":"MASCAB01A", "sessionId":"test","time":"2026-05-05 15:24:35", “Door1”:"Close",”Light”:"Off",”Lock1”:"Unlock","IP_ETH0":"192.168.137.101"} |
| Reboot Cabinet | in | {"cmd":"Reboot","sessionId":"test","mac":"MASCAB01A"} |
| After Reboot Reader Start Up | out | out {"cmd":"Scanning","hostname":"MASCAB01A","mac":"MASCAB01A","sessionId":"test"} |
| NowStatus | in | {"cmd":"NowStatus","sessionId":"test","mac":"all"} |
| - Response | out | {"mac":"MASCAB01C","macAddress":"b8:27:eb:0d:4c:cd","hostname":"MASCAB01C", "sessionId":"test","time":"2026-05-08 12:17:30",”Door1”:"Close",”Light”:"On",”Lock1”:"Lock","IP_ETH0":"192.168.137.103"} |
|  | out | {"mac":"MASCAB01B","macAddress":"b8:27:eb:6b:db:1e","hostname":"MASCAB01B", "sessionId":"test","time":"2026-05-08 12:17:30",”Door1”:"Close",”Light”:"On",”Lock1”:"Lock","IP_ETH0":"192.168.137.102"} |

# 8. Notes & Operational Considerations

- All communication is handled via RabbitMQ using the WebSocket protocol with endpoint ws://rabbitaddress:15675/ws.

- Input topics (commands to devices) use the key "in"; output topics (data from devices) use the key "out".

- To target a single cabinet, use its specific MAC address in the "mac" field. To broadcast to all cabinets, use "mac":"all".

- Ensure the RabbitMQ broker service is running and accessible on port 15675 (WebSocket) and port 15672 (Management UI) before starting the React application.

- Always restart the React application after modifying .env configuration variables.

- The routing key pattern for per-device subscriptions follows the format: {mac_address}.{topic} (e.g., b8:27:eb:6b:db:1e.door).

# Document Control

| Field | Details |
| --- | --- |
| Document Title | Cabinet Status Management System — Technical Reference |
| Version | 1.0.0(1) |
| Initiated By | M. Rahim |
| Organization | Mobile Aspects |
| Date | May 2026 |
| Review Cycle | As required upon system changes |

This document is the property of Mobile Aspects and is intended for internal use only. Unauthorized distribution or reproduction is prohibited.