MOBILE ASPECTS

Raspberry Pi Watchdog Auto-Recovery Setup

Technical Reference Documentation

| Document Version | Version 1.0.0(1) |
| --- | --- |
| Date | December 2025 |
| Initiated By | M. Rahim |
| Organization | Mobile Aspects |
| Classification | Confidential – Internal Use Only |
| Status | Draft |

Raspberry Pi Watchdog Auto-Recovery Setup

Install Watchdog Package:

sudo apt update

sudo apt install watchdog

Activate the Kernel Watchdog Driver

echo "bcm2835_wdt" | sudo tee -a /etc/modules

sudo modprobe bcm2835_wdt

Verify:

lsmod | grep wdt

Open the watchdog configuration file:

sudo nano /etc/watchdog.conf

Uncomment or add the following parameters:

watchdog-device = /dev/watchdog

max-load-1 = 24

temperature-device = /sys/class/thermal/thermal_zone0/temp

max-temperature = 75

ping = 8.8.8.8

ping = 1.1.1.1

retry-timeout = 60

Activate and start the service:

sudo systemctl enable watchdog

sudo systemctl start watchdog

Document Control

| Field | Details |
| --- | --- |
| Document Title | Raspberry Pi Watchdog Auto-Recovery Setup — Technical Reference |
| Version | 1.0.0(1) |
| Initiated By | M. Rahim |
| Organization | Mobile Aspects |
| Date | December 2025 |
| Review Cycle | As required upon system changes |

This document is the property of Mobile Aspects and is intended for internal use only. Unauthorized distribution or reproduction is prohibited.