MOBILE ASPECTS

Policy Web Utility

Technical Reference Documentation

| Document Version | Version 1.0.0(1) |
| --- | --- |
| Date | July 2026 |
| Initiated By | M. Rahim |
| Organization | Mobile Aspects |
| Classification | Confidential – Internal Use Only |
| Status | Draft |

# 1. Objective

This document outlines the procedure for reading and replacing cabinet policy (configuration) values through the Policy Web Utility. It is intended to serve as the definitive technical reference for developers, administrators, and operators within Mobile Aspects who are responsible for the deployment, configuration, and ongoing maintenance of the cabinet policy system.

# 2. Scope

This document applies to all Mobile Aspects personnel and developers responsible for the deployment, maintenance, and monitoring of cabinet policy values via the Policy Web Utility. The utility is composed of two cooperating services:

- A Windows-hosted service (module.py) that serves the “Policy Web” browser page and proxies requests to each cabinet's Raspberry Pi.

- A Raspberry Pi-hosted service (apps.py) running on each cabinet, which stores and applies the actual policy values.

# 3. Architecture Overview

The Policy Web Utility follows a Windows-to-Raspberry Pi proxy model. The Windows host runs the operator-facing web page; the Raspberry Pi inside each cabinet runs the policy storage/API service. Communication between the two occurs over plain HTTP on the local cabinet network.

## 3.1  Component Map

| Component | File | Host | Port | Role |
| --- | --- | --- | --- | --- |
| Policy Web (UI) | module.html | Windows | 5100 | Operator-facing page — Read / Write policy, change Windows hostname |
| Policy Web (backend) | module.py | Windows | 5100 | Flask app; resolves cabinet → RPi IP, proxies Read/Write calls |
| Windows-side config | module.json | Windows | — | Stores ip_broker used by module.py |
| Cabinet Service (UI) | index.html | Raspberry Pi | 8082 | On-cabinet dashboard showing current policy values |
| Cabinet Service (backend) | apps.py | Raspberry Pi | 8082 | Flask/Waitress app; stores policy in config.json, exposes /api/config |

## 3.2  Cabinet → Raspberry Pi IP Resolution

module.py resolves a Raspberry Pi's IP address directly from the cabinet ID, without a lookup table. The pattern is:

CAB<number><letter> --> 192.168.137.<number*100 + letter_value> letter_value: A=1, B=2, C=3, ... (1 if no letter suffix)

Examples:

- MASCAB01A  →  192.168.137.101

- MASCAB01B  →  192.168.137.102

The Raspberry Pi service is then reached at http://<resolved-ip>:8082.

# 4. Pre-requisites

Before proceeding with configuration, ensure the following prerequisites are satisfied:

- module.py is running on the Windows host and reachable on port 5100.

- apps.py is running on the target Raspberry Pi(s) and reachable on port 8082.

- module.json on the Windows host contains a valid ip_broker entry.

- The Windows host and the Raspberry Pi(s) are on the same network segment (192.168.137.x).

- At least one cabinet is detected/registered so it appears in the Select Cabinet dropdown.

# 5. Responsibilities

| Role | Responsibility |
| --- | --- |
| Developers | Implement, test, and debug module.py, apps.py, and their respective web pages. |
| Administrators | Monitor service availability on ports 5100 and 8082, manage policy values, and ensure data integrity in config.json / module.json. |
| Operators | Use the Policy Web page to read current policy values and submit policy changes for each cabinet. |

# 6. Endpoint Reference

The following endpoints are used to read and replace policy values.

## 6.1  Raspberry Pi Service (apps.py) — Port 8082

| Method | Endpoint | Purpose |
| --- | --- | --- |
| GET | /api/config | Returns the current policy values stored in config.json. |
| POST | /api/config | Replaces one or more policy fields and applies them. |
| POST | /api/reboot | Reboots the Raspberry Pi after a short delay. |
| GET | /api/status | Returns basic service/connection status. |
| POST | /api/auto-detect | Triggers auto-detection of the Windows broker IP. |

## 6.2  Windows Service (module.py) — Port 5100

| Method | Endpoint | Purpose |
| --- | --- | --- |
| GET | /rpi/config/<cabinet_id> | Proxies a Read request to the cabinet's Raspberry Pi (/api/config). |
| POST | /rpi/config/<cabinet_id> | Proxies a raw Write request to the cabinet's Raspberry Pi (/api/config). |
| POST | /send | Front-end “Write” action; routes a single policy option (hostname, location, ip_broker, reboot, logger, etc.) to the correct cabinet. |
| POST | /change_hostname | Changes the Windows host's own hostname and updates all related cabinets. |
| GET | /api/broker | Returns the currently configured broker IP. |
| POST | /set_broker | Manually sets the Windows-side broker IP ({ "ip_broker": "<ip>" }) and re-publishes it via MQTT to all cabinets. |

## 6.3  Policy Fields

| Field | Description |
| --- | --- |
| hostname | Cabinet device hostname. |
| location | Free-text location label for the cabinet. |
| ip_broker | IP address of the RabbitMQ/MQTT broker the cabinet should use. |
| rfid_filter | Comma-separated tag-prefix filter (stored as a list), e.g. E2, 64, 30. |
| scan_timeout | Scan cutoff time in seconds (numeric). |
| scan_newtag | No-new-tag limit in seconds (numeric). |
| PORT | Serial port used by the RFID reader (read-only via this page). |
| logger | enable or disable — controls whether log files are written. |
| reboot | Not a stored field; triggers an immediate reboot of the cabinet's Raspberry Pi. |
| rf_mode | RFID reader RF mode/profile code (e.g. DC, CB, 6F). Determines read rate vs. sensitivity trade-off. Applied on the next reboot. |
| power | RFID reader TX power in dBm (25-33). Applied on the next reboot. |
| antenna | List of enabled antenna IDs (1-5), e.g. ["1","2","3","4"]. Applied on the next reboot. |

## 6.4  REST API Response Reference

This section consolidates the example responses, accepted fields, and error/response codes for the core policy endpoint, GET/POST /api/config on apps.py (Raspberry Pi). The same response codes apply to the equivalent proxy calls on module.py.

GET /api/config — Example Response

{ "location": "RAP1", "hostname": "MASCAB01A", "rfid_filter": ["E2","64"], "scan_timeout": 120, "scan_newtag": 5, "rf_mode": "DC", "power": 30, "antenna": ["1","2","3","4"], "ip_broker": "172.203.216.202", "PORT": "/dev/ttyUSB0", "logger": "enable" }

POST /api/config — Supported Fields

hostname, location, ip_broker, rfid_filter, scan_timeout, scan_newtag, rf_mode, power, antenna, PORT, logger — any field not in this list is silently ignored.

POST /api/config — Success Response (HTTP 200)

{ "status": "ok", "updated": ["location", "scan_timeout"] }

updated lists only the fields that were actually recognised and applied from the request body.

Error Responses

| Endpoint | Code | Message |
| --- | --- | --- |
| GET/POST /api/config (apps.py) | 400 | "Request body is empty" |
| GET/POST /api/config (apps.py) | 400 | "scan_timeout must be a number" |
| GET/POST /api/config (apps.py) | 400 | "scan_newtag must be a number" |
| GET/POST /api/config (apps.py) | 400 | "No recognised fields in request" |
| GET/POST /rpi/config/ <cabinet_id> (module.py) | 400 | "Tidak bisa resolve IP untuk '<cabinet_id>'" |
| GET/POST /rpi/config/ <cabinet_id> (module.py) | 503 | "RPi <ip> Unreachable (port 8082)" |
| GET/POST /rpi/config/ <cabinet_id> (module.py) | 504 | "RPi <ip> timeout" |
| POST /send (module.py) | 400 | "option empty" / "cabinet ID empty" / "Unknown option: <option>" / field-specific validation messages |
| POST /send ip_broker broadcast (module.py) | 207 | Partial success — reached some but not all cabinets on the old broker |
| POST /change_hostname (module.py) | 400 | "Hostname empty" |
| POST /set_broker (module.py) | 400 | "ip_broker is required" |
| GET/POST /api/config (apps.py) | 400 | "rf_mode must be one of: <valid codes>" |
| GET/POST /api/config (apps.py) | 400 | "power must be between 25 and 33 dBm" |
| GET/POST /api/config (apps.py) | 400 | "antenna must be one or more valid IDs (1-5)" |

HTTP Response Codes

| HTTP Code | Meaning |
| --- | --- |
| 200 | Success |
| 207 | Partial success — only used by the ip_broker broadcast in POST /send, when some cabinets accepted the change and others did not |
| 400 | Invalid payload — missing/empty value, wrong type, or unrecognised option |
| 500 | Internal error, or the Raspberry Pi rejected the request (e.g. reboot or send failure) |
| 503 | Raspberry Pi unreachable on port 8082 (connection refused / host down) |
| 504 | Raspberry Pi did not respond within the request timeout |

# 7. Reading Policy

Policy values can be read either directly from the Raspberry Pi (on-cabinet dashboard) or remotely from the Windows Policy Web page.

## 7.1  Read via Windows Policy Web Page

- Open the Policy Web page in a browser at http://<windows-host>:5100/.

- Under Card 1, select the target cabinet from the Select Cabinet dropdown.

- Click the Read toggle button.

- The browser calls GET /rpi/config/<cabinet_id> on module.py (port 5100). module.py resolves the cabinet ID to a Raspberry Pi IP, forwards the request as GET /api/config to apps.py (port 8082), and returns the JSON response to the browser. The full request/response chain and the UI field mapping are detailed in the tables below.

Switching cabinets while in Read mode automatically re-reads policy for the newly selected cabinet.

What is sent when Read is clicked — full request/response chain

The following table traces the exact HTTP calls triggered by clicking the Read button on the Windows Policy Web page, using cabinet MASCAB01J as the example (RPI IP 192.168.137.110).

| Step | Who → Who | HTTP Call & Notes |
| --- | --- | --- |
| 1 | Browser → module.py | GET http://<windows-host>:5100/rpi/config/MASCAB01J No request body. Triggered when user clicks the Read button, or switches the Select Cabinet dropdown while in Read mode. |
| 2 | module.py → apps.py | GET http://192.168.137.110:8082/api/config module.py resolves MASCAB01J to its RPI IP (192.168.137.110) from the cabinet list. No request body. |
| 3 | apps.py → module.py | HTTP 200 — JSON body (config.json contents of that Raspberry Pi): { "location":"MASCAB01", "hostname":"MASCAB01J", "rfid_filter":["E2","64"], "scan_timeout":20, "scan_newtag":5, "rf_mode":"DC", "power":30, "antenna":["1","2","3","4"], "ip_broker":"192.168.137.1", "PORT":"ttyS0", "logger":"enable" } |
| 4 | module.py → Browser | Same JSON body + one extra field injected by module.py: "_rpi_ip": "192.168.137.110" ← actual IP used for the request HTTP 200. |
| 5 | Browser renders | module.html JavaScript receives the JSON and maps each key to a display row (see UI Label Mapping table below). |

UI Label → JSON Key Mapping

The JSON response from apps.py uses different key names from the labels shown on screen. The table below shows the exact mapping, using the values visible in the screenshot above.

| UI Label (displayed) | JSON Key | Example Value | How it is Rendered |
| --- | --- | --- | --- |
| HOSTNAME | hostname | "MASCAB01J" | Plain text value |
| LOCATION | location | "MASCAB01" | Plain text value |
| SERVER IP | ip_broker | "192.168.137.1" | Plain text value. Note: JSON key is ip_broker, NOT server_ip |
| TAG FILTER | rfid_filter | ["E2","64"] | Each array item rendered as a separate coloured chip/badge (e.g. E2 64). Empty array shows “—” |
| SCAN CUTOFF (S) | scan_timeout | 20 | Numeric, plain text |
| NONEWTAGLIMIT (S) | scan_newtag | 5 | Numeric, plain text |
| SERIAL PORT | PORT | "ttyS0" | Plain text. Read-only — cannot be changed from the Policy Web page |
| LOGGER | logger | "enable" | "enable" → green ● ENABLED badge "disable" → red ● DISABLED badge |
| RPI IP (footer) | _rpi_ip | "192.168.137.110" | Displayed in small text at the bottom-right of the config card. Added by module.py; not in apps.py’s own response. |
| RF MODE | rf_mode | "DC" | Plain text value. Changing this requires a Raspberry Pi reboot to take effect. |
| TX POWER | power | 30 | Plain text value (dBm). Changing this requires a Raspberry Pi reboot to take effect. |
| ANTENNA | antenna | ["1","2","3","4"] | Each antenna ID rendered as a separate chip. Changing this requires a Raspberry Pi reboot to take effect. |

## 7.2  Read Directly on the Raspberry Pi

The on-cabinet dashboard (index.html, served by apps.py) shows the same policy values locally. It loads them on page open via JavaScript calling GET /api/config, and exposes a Refresh button to reload the values.

GET http://<raspberry-pi-ip>:8082/api/config Response (example): { "location": "RAP1", "hostname": "MASCAB01A", "rfid_filter": ["E2","64"], "scan_timeout": 120, "scan_newtag": 5, "rf_mode": "DC", "power": 30, "antenna": ["1","2","3","4"], "ip_broker": "172.203.216.202", "PORT": "/dev/ttyUSB0", "logger": "enable" }

## 7.3  Complete Read Field Reference

Read calls are plain GET requests with no request body and no parameters other than the cabinet ID in the URL. There are two request shapes, depending on which page is used:

A) Via Windows Policy Web Page (proxied through module.py)

Step 1 — Browser to module.py: GET http://<windows-host>:5100/rpi/config/<cabinet_id> (no request body) Step 2 — module.py forwards to apps.py: GET http://<rpi-ip>:8082/api/config (no request body)

B) Via Raspberry Pi Dashboard (direct, no module.py involved)

GET http://<raspberry-pi-ip>:8082/api/config (no request body)

Both requests return the same JSON object from apps.py (path A adds one extra field, _rpi_ip). The table below maps each displayed field to its JSON key, an example value, and the default used when the field is absent from config.json.

| Displayed Label | JSON Key | Example Value | Default (if missing) | Notes |
| --- | --- | --- | --- | --- |
| Hostname | hostname | "MASCAB01A" | "" (empty string) | — |
| Location | location | "RAP1" | "" (empty string) | — |
| Server IP | ip_broker | "172.203.216.202" | "" (empty string) | Stored in the same config.json on the Raspberry Pi as the other fields. |
| Tag Filter | rfid_filter | ["E2","64"] | [ ] (empty array) | Rendered as colored chips; an empty list shows “No filter” / “(kosong)”. |
| Scan Cutoff (s) | scan_timeout | 120 | 120 | — |
| noNewTagLimit (s) | scan_newtag | 5 | 5 | — |
| Serial PORT | PORT | "/dev/ttyUSB0" | "" (empty string) | Read-only on both UIs; not present in the Change Policy dropdown. |
| Logger | logger | "enable" | "enable" | Rendered as a green ENABLED / red DISABLED badge on both UIs. |
| RPi IP (Windows page only) | _rpi_ip | "192.168.137.103" | — | Added by module.py after proxying the request; not part of apps.py's own response, and not shown on the RPi dashboard. |
| RF Mode | rf_mode | "DC" | "DC" | Applied at reader startup only — a reboot is required for a changed value to take effect. |
| TX Power (dBm) | power | 30 | 30 | Applied at reader startup only — a reboot is required for a changed value to take effect. |
| Antenna | antenna | ["1","2","3","4"] | ["1","2","3","4"] | Applied at reader startup only — a reboot is required for a changed value to take effect. |

# 8. Replacing (Writing) Policy

Policy values are replaced by submitting one field at a time from the Windows Policy Web page, which forwards the change to the target Raspberry Pi.

## 8.1  Write via Windows Policy Web Page

- Open the Policy Web page and select the target cabinet.

- Ensure the Write toggle button is active (this is the default mode).

- Choose the policy to change from the Change Policy dropdown: Hostname, Location, Server IP, Reboot, Logger, Tag Filter, Scan Cutoff, noNewTagLimit, RF Mode, TX Power, or Antenna Enable.

- Enter the new value (or choose enable/disable for Logger; no value is required for Reboot).

- Click Send.

Behind the scenes, the page calls POST /send on module.py with the selected mac, option, and value. module.py then resolves the cabinet's Raspberry Pi IP and forwards the change to POST http://<rpi-ip>:8082/api/config (or /api/reboot for the Reboot option).

## 8.2  Complete Write Field Reference

Every option in the Change Policy dropdown is submitted from the browser the same way: a single POST /send call to module.py with the cabinet ID, the option key, and the value. module.py then validates/transforms that value and forwards it to the target Raspberry Pi as a second HTTP call. The request shape from the browser is always:

POST http://<windows-host>:5100/send Content-Type: application/json { "mac": "<cabinet_id>", "option": "<option key>", "value": "<value>" }

The <option key> matches the Field column in section 6.3. The table below lists, for each Change Policy item: what to enter as the value, which apps.py endpoint module.py forwards the change to, the exact forwarded payload, and any special behavior.

| Change Policy Option | Value to Enter | Forwarded To (apps.py) | Forwarded Payload (JSON) | Notes |
| --- | --- | --- | --- | --- |
| Hostname (option: hostname) | New device hostname, e.g. MASCAB01A | POST http://<rpi-ip>:8082 /api/config | { "hostname": "MASCAB01A" } | RPi applies the new system hostname, then auto-reboots (~3s delay) to apply it. |
| Location (option: location) | Free-text location label, e.g. SUPPLY7 | POST http://<rpi-ip>:8082 /api/config | { "location": "SUPPLY7" } | No reboot triggered. |
| Server IP (option: ip_broker) | Broker IP address, e.g. 172.203.216.202 | POST http://<rpi-ip>:8082 /api/config — sent to EVERY cabinet on the OLD broker (broadcast), not just the selected cabinet | { "ip_broker": "172.203.216.202" } | module.py also saves the value to module.json on Windows and switches CURRENT_BROKER. This path does not use MQTT. |
| Reboot (option: reboot) | None — no value field required | POST http://<rpi-ip>:8082 /api/reboot | { } (empty JSON body) | Does not modify config.json; only triggers the reboot endpoint. |
| Logger (option: logger) | enable or disable (chosen from dedicated dropdown) | POST http://<rpi-ip>:8082 /api/config | { "logger": "enable" } | module.py rejects any value other than enable/disable with HTTP 400 before forwarding. |
| RF Mode (option: rf_mode) | RF mode code, e.g. DC | POST http://<rpi-ip>:8082 /api/config | { "rf_mode": "DC" } | Validated against reader.py's RF_MODE_TABLE codes; applied only after the next reboot. |
| TX Power (option: power) | Power in dBm, e.g. 30 | POST http://<rpi-ip>:8082 /api/config | { "power": 30 } | Must be between 25 and 33 dBm; applied only after the next reboot. |
| Antenna Enable (option: antenna) | Comma-separated antenna IDs, e.g. 1,3 | POST http://<rpi-ip>:8082 /api/config | { "antenna": ["1","3"] } | Any combination of antenna IDs 1-5 is accepted; applied only after the next reboot. |
| Tag Filter (option: rfid_filter) | Comma-separated prefixes, e.g. E2,64,30 | POST http://<rpi-ip>:8082 /api/config | { "rfid_filter": ["E2","64","30"] } | module.py splits the text on commas, upper-cases each tag, and sends it as a JSON array (not a string). |
| Scan Cutoff(second) (option: scan_timeout) | Number of seconds, e.g. 120 | POST http://<rpi-ip>:8082 /api/config | { "scan_timeout": 120 } | Converted to an integer; non-numeric input is rejected with HTTP 400 before forwarding. |
| noNewTagLimit(second) (option: scan_newtag) | Number of seconds, e.g. 5 | POST http://<rpi-ip>:8082 /api/config | { "scan_newtag": 5 } | Converted to an integer; non-numeric input is rejected with HTTP 400 before forwarding. |

## 8.3  Write Request Payload Examples

// Change hostname POST /send { "mac":"MASCAB01A", "option":"hostname", "value":"MASCAB01A" } // Change location POST /send { "mac":"MASCAB01A", "option":"location", "value":"SUPPLY7" } // Change RFID tag filter POST /send { "mac":"MASCAB01A", "option":"rfid_filter", "value":"E2,64,30" } // Enable or disable logger POST /send { "mac":"MASCAB01A", "option":"logger", "value":"enable" } // Change broker (Server) IP -- broadcasts to all cabinets on the old broker POST /send { "mac":"MASCAB01A", "option":"ip_broker", "value":"172.203.216.202" } // Change scan cutoff (seconds) POST /send { "mac":"MASCAB01A", "option":"scan_timeout", "value":"120" } // Change noNewTagLimit (seconds) POST /send { "mac":"MASCAB01A", "option":"scan_newtag", "value":"5" } // Change RF mode POST /send { "mac":"MASCAB01A", "option":"rf_mode", "value":"DC" } // Change TX power (dBm) POST /send { "mac":"MASCAB01A", "option":"power", "value":"30" } // Change antenna selection POST /send { "mac":"MASCAB01A", "option":"antenna", "value":"1,2,3,4" } // Reboot the cabinet POST /send { "mac":"MASCAB01A", "option":"reboot" }

## 8.4  Direct Replacement on the Raspberry Pi

Policy can also be written directly to a Raspberry Pi without going through module.py, by posting to its /api/config endpoint:

POST http://<raspberry-pi-ip>:8082/api/config Content-Type: application/json { "scan_timeout": 90, "scan_newtag": 10 } Response: { "status": "ok", "updated": ["scan_timeout", "scan_newtag"] }

Only fields listed in section 6.3 are accepted; unrecognised fields are ignored. If hostname is changed, the Raspberry Pi schedules a reboot to apply the new system hostname. If ip_broker is changed, the broker/bridge configuration is re-applied in the background.

## 8.5  Special Cases

- ip_broker (sent via /send): broadcasts the new broker IP to every cabinet currently known to the old broker, not only the selected cabinet.

- hostname: updates the cabinet's system hostname and triggers an automatic reboot of the Raspberry Pi.

- reboot: does not modify config.json; it only triggers POST /api/reboot.

- logger: accepts only enable or disable; any other value is rejected with an error.

- rf_mode, power, antenna: reader.py only reads these values at process startup, so apps.py automatically reboots the Raspberry Pi after saving any of these three fields for the new value to take effect.

# 9. Notes & Operational Considerations

- All Read/Write calls from the Windows Policy Web page require the target cabinet's Raspberry Pi to be reachable on port 8082; if unreachable, the page reports “RPi <ip> Unreachable (port 8082)”.

- Policy changes are written to config.json on the Raspberry Pi and to module.json (for ip_broker) on the Windows host.

- Switching the selected cabinet while in Read mode automatically triggers a fresh read; no manual refresh is required.

- The Update Windows Hostname card (Card 2 of the Policy Web page) is a separate function from cabinet policy: it changes the Windows host's own hostname and cascades a matching location update to all detected cabinets.

- Always confirm the Raspberry Pi service (apps.py) has fully restarted before re-reading policy after a hostname or broker change, since both changes restart the underlying service.

# 10. Manual Testing via PowerShell / CMD

This section provides ready-to-run PowerShell commands for manually testing Write (POST) and Read (GET) operations against both the Windows service (module.py, port 5100) and the Raspberry Pi service (apps.py, port 8082). Use these to verify each policy field without opening the browser UI.

## 10.1  Write Commands (POST) via PowerShell

All Write calls use Invoke-RestMethod. Replace MASCAB01J with the target cabinet ID and 192.168.137.1 with your Windows host IP where applicable.

| Policy Field | PowerShell Command | JSON Body Sent |
| --- | --- | --- |
| Logger | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"logger","value":"enable"}' | {"mac":"MASCAB01J", "option":"logger", "value":"enable"} |
| Logger | Invoke-RestMethod -Uri "http://192.168.137.110:8082/api/config" -Method POST -ContentType "application/json" -Body '{"logger":"enable"}' | {"logger":"enable"} |
| Hostname | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"hostname","value":"MASCAB01J"}' | {"mac":"MASCAB01J", "option":"hostname", "value":"MASCAB01J"} |
| Location | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"location","value":"MASCAB01"}' | {"mac":"MASCAB01J", "option":"location", "value":"MASCAB01"} |
| Server IP (ip_broker) | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"ip_broker","value":"192.168.137.1"}' | {"mac":"MASCAB01J", "option":"ip_broker", "value":"192.168.137.1"} |
| Tag Filter (rfid_filter) | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"rfid_filter","value":"E2,64"}' | {"mac":"MASCAB01J", "option":"rfid_filter", "value":"E2,64"} |
| Scan Cutoff (scan_timeout) | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"scan_timeout","value":"20"}' | {"mac":"MASCAB01J", "option":"scan_timeout", "value":"20"} |
| noNewTagLimit (scan_newtag) | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"scan_newtag","value":"5"}' | {"mac":"MASCAB01J", "option":"scan_newtag", "value":"5"} |
| Reboot | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"reboot"}' | {"mac":"MASCAB01J", "option":"reboot"} (no value field) |
| RF Mode | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"rf_mode","value":"DC"}' | {"mac":"MASCAB01J", "option":"rf_mode", "value":"DC"} |
| TX Power (power) | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"power","value":"30"}' | {"mac":"MASCAB01J", "option":"power", "value":"30"} |
| Antenna Enable (antenna) | Invoke-RestMethod -Uri "http://localhost:5100/send" -Method POST -ContentType "application/json" -Body '{"mac":"MASCAB01J","option":"antenna","value":"1,2,3,4"}' | {"mac":"MASCAB01J", "option":"antenna", "value":"1,2,3,4"} |

## 10.2  Read Commands (GET) via PowerShell

Read calls are plain GET requests with no request body. Two paths are available depending on whether you go through module.py or directly to the RPi.

| Read Path | PowerShell Command | Notes |
| --- | --- | --- |
| Via module.py (Windows → RPi proxy) | Invoke-RestMethod -Uri "http://localhost:5100/rpi/config/MASCAB01J" -Method GET | Returns all policy fields + extra _rpi_ip field injected by module.py. No request body needed. |
| Direct to RPi (apps.py) | Invoke-RestMethod -Uri "http://192.168.137.110:8082/api/config" -Method GET | Replace 192.168.137.110 with the actual RPi IP. Returns policy fields only (no _rpi_ip). No request body needed. |

Note: PowerShell does not support the bare POST <url> { ... } syntax. Always use Invoke-RestMethod with -Method POST and -Body as shown above.

# Document Control

| Field | Details |
| --- | --- |
| Document Title | Policy Web Utility — Technical Reference |
| Version | 1.0.0(1) |
| Initiated By | M. Rahim |
| Organization | Mobile Aspects |
| Date | July 2026 |
| Review Cycle | As required upon system changes |

This document is the property of Mobile Aspects and is intended for internal use only. Unauthorized distribution or reproduction is prohibited.